Cisco Router - DHCP Relay Configuration

Configuring DHCP Relay on a Cisco Router

In networks where the DHCP server is located on a different subnet than the clients, you need to configure DHCP Relay. DHCP Relay allows a router to forward DHCP requests from clients in one subnet to a DHCP server in another subnet, ensuring all devices can obtain IP addresses automatically, even if they’re not on the same subnet as the DHCP server.

Why Use DHCP Relay?

- Centralized DHCP Server: You can have a single DHCP server providing IP addresses to multiple subnets, reducing the need for multiple DHCP servers.

- Simplified Network Management: Allows network administrators to manage IP allocation from a central location.

- Efficient Use of IPs: It ensures that clients in various subnets can receive IP addresses from one central DHCP server, promoting efficient use of resources.

How DHCP Relay Works

When a client sends a DHCP request (a broadcast message), the router configured with DHCP Relay receives that request. The router forwards the request to the DHCP server, appending its interface address to let the server know which network/subnet the request originated from. The DHCP server then replies, and the router forwards the response back to the client.

Here’s how to set up DHCP Relay on a Cisco router.

Step 1: Enter Global Configuration Mode

- Router> enable

- Router# configure terminal

Step 2: Enter the Interface Configuration Mode

You need to configure DHCP Relay on the interface connected to the clients. Enter the interface configuration mode:

- Router(config)# interface GigabitEthernet0/1

Replace GigabitEthernet0/1 with the appropriate interface on your router.

Step 3: Configure the DHCP Relay Agent

Tell the router where to forward DHCP requests by specifying the IP address of the DHCP server.

- Router(config-if)# ip helper-address 192.168.2.100

Here, 192.168.2.100 is the IP address of the DHCP server located on a different subnet.

Troubleshooting DHCP Relay

- Verify Helper Address: Ensure that the correct DHCP server IP address is configured as the helper address.

- Check Firewall Rules: Ensure that UDP traffic on ports 67 (DHCP server) and 68 (DHCP client) is allowed across the network.

- Relay Agent IP Address: Confirm that the relay agent is adding the correct interface IP address to inform the DHCP server which subnet the request came from.