DHCP Snooping Configuration

Introduction

In modern networks, the Dynamic Host Configuration Protocol (DHCP) plays a crucial role in automatically assigning IP addresses and other network configurations to devices. However, DHCP is vulnerable to various security threats, including DHCP spoofing. This article delves into DHCP spoofing, its implications, and how to mitigate it using Cisco network devices

What is DHCP Spoofing?

DHCP spoofing is a type of attack where a malicious actor sets up a rogue DHCP server on a network. The rogue server can respond to DHCP requests from clients with false information, such as incorrect IP addresses, DNS servers, or gateways. This can lead to several issues, including:

- Man-in-the-Middle Attacks: The attacker can intercept and manipulate the traffic between clients and legitimate servers.

- Denial of Service (DoS): By assigning incorrect network configurations, the attacker can disrupt normal network operations.

- Network Misconfiguration: Clients may receive incorrect network settings, causing communication failures within the network.

How DHCP Spoofing Works

In a typical network setup, DHCP clients broadcast a DHCP Discover message to locate a DHCP server. Both the legitimate and rogue DHCP servers can respond to this message with a DHCP Offer. If the client accepts the offer from the rogue server, it will receive potentially harmful network configurations.

Mitigating DHCP Spoofing with Cisco DHCP Snooping

Cisco provides a robust solution to combat DHCP spoofing attacks: DHCP Snooping. DHCP Snooping acts as a firewall between untrusted hosts and the trusted DHCP server. Here's how to configure it:

Enable DHCP Snooping Globally

- DMT-SW01(config)# ip dhcp snooping

Enable DHCP Snooping on Specific VLANs

- DMT-SW01(config)# ip dhcp snooping vlan 10,20

Define Trusted and Untrusted Ports

By default, all ports are untrusted. Trusted ports are those connected to legitimate DHCP servers.

Marking a trusted port:

- DMT-SW01(config)# interface gigabitEthernet 0/1

- DMT-SW01(config-if)# ip dhcp snooping trust

Set the Rate Limit for DHCP Packets on Untrusted Ports

This limits the number of DHCP packets an untrusted port can send per second, mitigating DoS attacks.

- DMT-SW01(config)# interface range fastEthernet 0/1-24

- DMT-SW01(config-if-range)# ip dhcp snooping limit rate 15

Verify DHCP Snooping Configuration

After configuration, verify that DHCP Snooping is active and working correctly.

- DMT-SW01# show ip dhcp snooping

Example Configuration

Here's an example configuration on a Cisco switch for VLANs 10 and 20, with ports configured as trusted or untrusted:

- DMT-SW01(config)# ip dhcp snooping

- DMT-SW01(config)# ip dhcp snooping vlan 10,20

- DMT-SW01(config)# interface gigabitEthernet 0/1

- DMT-SW01(config-if)# ip dhcp snooping trust

- DMT-SW01(config)# interface range fastEthernet 0/1-24

- DMT-SW01(config-if-range)# ip dhcp snooping limit rate 15

Monitoring and Troubleshooting

To ensure DHCP Snooping is functioning correctly, use the following command to view detailed information about the DHCP Snooping status, trusted ports, and rate limits:

- DMT-SW01# show ip dhcp snooping

This output provides insight into which VLANs are protected, the status of the snooping database, and any statistics related to DHCP packet rates.

Conclusion

DHCP spoofing is a serious threat that can disrupt network operations and compromise security. By implementing DHCP Snooping on Cisco devices, network administrators can effectively mitigate these risks. This security measure, combined with regular monitoring and proper network configuration, ensures that DHCP operations remain secure and reliable. Implementing DHCP Snooping is a critical step in maintaining a robust and secure network infrastructure.