Route Maps

In dynamic routing environments, it’s essential to control which routes are advertised and accepted to ensure efficient routing, security, and traffic flow. Two powerful tools available for this purpose in Cisco devices are Route Maps and Distribute Lists. These tools give network administrators fine-tuned control over routing behavior by filtering, modifying, and selectively allowing routes to be advertised or received.

This article will break down what Route Maps and Distribute Lists are, how they function, and how to use them to control routes in your network.

What are Route Maps?

Route Maps are a flexible and powerful tool in Cisco’s routing arsenal. Think of route maps as conditional statements, allowing you to match specific criteria and then apply actions based on those conditions. They work similarly to an "if-then" logic: "If a route matches this condition, then apply this action."

Route Maps are often used in conjunction with routing protocols like EIGRP, OSPF, and BGP to control routing decisions and influence traffic flow.

Key Components of a Route Map:

- Match Clause: This specifies the criteria a route must meet, such as matching an IP address, prefix length, or AS path.

- Set Clause: This defines the actions that will be applied to the route if it matches the criteria, such as setting a metric, modifying a route tag, or applying a specific weight.

- Permit or Deny: The final outcome of the route map, where permit allows the route, and deny blocks it.

Example of a Route Map:

Let’s say you want to match all traffic going to the 192.168.0.0/24 network and then increase the metric, making the route less preferable:

route-map CHANGE-METRIC permit 10

match ip address 10

set metric 100

In this example:

- The match ip address 10 references an access-list

- The set metric 100 command increases the metric, making this route less attractive.

By applying this route map to a routing protocol, you control how specific routes are treated.

What are Distribute Lists?

Distribute Lists are a simpler tool used to filter routing information. They can be used to control which routes are received or advertised by a router. Distribute lists use Access Control Lists (ACLs) or Prefix Lists to define which routes should be permitted or denied.

Distribute Lists are commonly used with protocols like RIP, EIGRP, and OSPF to control route propagation, limiting which routes enter the routing table or are advertised to neighbors.

Example of a Distribute List:

Let’s assume you want to block the network 10.4.4.0/24 from being advertised by your EIGRP process:

access-list 10 deny 10.4.4.0 0.0.0.255

access-list 10 permit any

router eigrp 1

distribute-list 10 out

Here’s what’s happening:

- access-list 10 denies the 10.4.4.0/24 network while permitting everything else.

- distribute-list 10 out is applied to the EIGRP process, blocking 10.4.4.0/24 from being advertised out.

Using Route Maps for Route Control

Route Maps offer much more flexibility compared to Distribute Lists. Not only can you match on IP addresses, but you can also match on metrics, next-hop addresses, route tags, or even BGP attributes. You can also modify routes by setting new metrics, next hops, or weight values, making Route Maps an incredibly versatile tool.

Example: Blocking a Specific Route from Being Advertised via EIGRP

Let’s say you want to prevent the network 10.5.5.0/24 from being advertised to your neighbors via EIGRP:

- Create an access list to match the network you want to block:

access-list 5 deny 10.5.5.0 0.0.0.255

access-list 5 permit any

- Create a route map that matches the ACL and denies the route:

route-map BLOCK-ROUTE deny 10

match ip address 5

- Apply the route map to your EIGRP process:

router eigrp 1

distribute-list route-map BLOCK-ROUTE out

In this example:

- The access-list 5 blocks the 10.5.5.0/24 network, while permitting all other networks.

- The route-map BLOCK-ROUTE deny 10 matches the ACL and prevents that route from being advertised.

Using Distribute Lists for Route Control

Distribute Lists provide a simpler way to filter routes based on IP addresses or prefixes. They are ideal when you only need basic route filtering without needing to modify route attributes like metrics or next hops.

Example: Filtering Routes with a Distribute List

Let’s suppose you want to prevent the 192.168.10.0/24 network from being received by your router via OSPF:

- Create an ACL that denies the network:

access-list 15 deny 192.168.10.0 0.0.0.255

access-list 15 permit any

- Apply the distribute list to your OSPF process:

router ospf 1

distribute-list 15 in

This configuration blocks the 192.168.10.0/24 network from being added to your routing table via OSPF, but permits all other routes.

Conclusion

Both Route Maps and Distribute Lists provide powerful mechanisms to control how routing information is processed within your network. While Distribute Lists are ideal for simple route filtering based on IP addresses or prefixes, Route Maps offer more flexibility by allowing you to match multiple criteria and modify route attributes. Understanding how to apply these tools effectively will give you greater control over your routing environment, allowing you to optimize traffic flow, improve security, and ensure that your network behaves as expected.