Stateless DHCPv6

Configuration Example: Stateless DHCPv6

Here’s how you configure Stateless DHCPv6:

Step 1: Configure the DHCPv6 Pool

As with stateful, create a DHCPv6 pool, but without specifying an address prefix:

- Router(config)# ipv6 dhcp pool DHCPv6_POOL

- Router(config-dhcpv6)# dns-server 2001:4860:4860::8888

- Router(config-dhcpv6)# domain-name example.com

Step 2: Enable the Interface for Stateless DHCPv6

Apply the pool to an interface:

- Router(config)# interface GigabitEthernet0/0

- Router(config-if)# ipv6 dhcp server DHCPv6_POOL

Step 3: Enable the Other Flag in RAs

Instruct hosts to use SLAAC for address assignment but DHCPv6 for additional configuration:

- Router(config-if)# ipv6 nd other-config-flag