Verifying the DHCP Configuration

You can verify the configuration using the following command:

- Router# show ip dhcp binding

This shows a list of all IP addresses assigned by the DHCP server.

You can also view DHCP configuration details:

- Router# show running-config | include dhcp

Troubleshooting Tips

- Check Excluded Addresses: Ensure that the range of excluded IP addresses doesn’t overlap with the DHCP pool.

- Verify Gateway and DNS Configuration: Double-check that the correct default gateway and DNS server addresses are being issued to clients.